6

PERFORM IP SLA FOR FOLLOWING TOPOLOGY

ROUTER 1 CONFIGURATION

- Configure terminal

- Interface fastEthernet 0/0

- Ip address 10.0.0.1 255.255.255.0

- no shutdown

- exit

SET ROUTING PROTOCOL

- router rip

- version 2

- network 10.0.0.0

- exit

CONFIGURING SLA

- ip sla 1

- icmp-echo 10.0.1.2 //(send echo messages to this ip address)

- timeout 1000

- frequency 3

- exit

- ip sla schedule 1 life forever start-time now

- track 1 ip sla 1 reachability

- exit

SET UP DEFAULT ROUTE

- ip route 0.0.0.0 0.0.0.0 10.0.0.2 track 1

- ip route 0.0.0.0 0.0.0.0 10.0.1.2 10

- exit

ROUTER 2 CONFIGURATION

- Configure terminal

- Interface fastEthernet 0/0

- Ip address 10.0.0.2 255.255.255.0

- no shutdown

- exit

- interface fastethernet 1/0

- ip address 10.0.1.1 255.255.255.0

- no shutdown

- exit

SET ROUTING PROTOCOL

- router rip

- version 2

- network 10.0.0.0

- network 10.0.1.0

- exit

ROUTER 3 CONFIGURATION

- Configure terminal

- Interface fastEthernet 0/0

- Ip address 10.0.1.2 255.255.255.0

- no shutdown

- exit

SET ROUTING PROTOCOL

- router rip

- version 2

- network 10.0.1.0

- exit

TO CHECK CONNECTION FROM R1

- show ip route

- show track

- show ip sla sta

- ping 10.0.0.2